Performed By: 					Date:

Reagents

|  | Supplier | Catalog No. | Lot No. |
| --- | --- | --- | --- |
| MaxPAR X8 | DVS |  |  |
| Neutral TCEP, 0.5M (10 l aliquots) | Pierce | 77720 |  |
| C Buffer | DVS |  |  |
| L Buffer | DVS |  |  |
| R Buffer | DVS |  |  |
| W Buffer | DVS |  |  |
| 3 kDa Amicon Ultra | Millipore | UFC500396 |  |
| 50 kDa Amicon Ultra | Millipore | UFC505096 |  |

Antibodies

| Antigen | Clone | Supplier | Cat No. | Lot No. | Input | Metal Label | Metal Conc. | Metal Vol. | Recovery (mg/ml) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 100 g |  | 0.05 M | 5 l |  |
|  |  |  |  |  | 200 g |  | 0.05 M | 10 l |  |
|  |  |  |  |  | 500 g |  | 0.05 M | 25 l |  |
|  |  |  |  |  | 1 mg |  | 0.05 M | 50 l |  |

Procedure

Always measure the starting concentration of your antibodies via Nanodrop – concentrations listed by vendors are often wrong. Use filter tips for all steps. All centrifugations are carried out at room temperature. Keep purified antibodies at 4 degrees C until step 5 and after finishing.

| MaxPar Polymer Tube | Antibody Tube |
| --- | --- |
| - Retrieve one 1x PCR tube of MaxPar per 100 g of antibody. Briefly spin before opening. |  |
| - For each antibody, add 95 l of L buffer to one MaxPar tube. Pipette up and down to mix well. Transfer the full 95 l to additional MaxPar tubes as necessary to obtain the proper amount of polymer (1 PCR tube per 100 g) in 95 l final volume. |  |
| - Add metal solution to MaxPar according to table. Vortex to mix. If available, record metal lot number in comments. |  |
| - Incubate MaxPar tube at room temperature for 60 minutes, vortexing every 20 minutes. Record times. - Start time: - Vortex 1: - Vortex 2: - Finish time: | - 35 minutes after start of incubation (step 4), pipette required amount of antibody (see table) into a labeled 50kDa filtration unit. If the volume exceeds 500 l, spin the filter at 12,000 x g for 8 minutes first to concentrate the antibody, repeating as necessary until all antibody is loaded. - Bring total volume to 400 l with R buffer. - Spin antibody filter units at 12,000 x g for 8 minutes. Discard flow-through. - Add 400 l of R buffer. - Spin antibody filter units at 12,000 x g for 8 minutes. Discard flow-through. - Dilute 0.5M TCEP by adding 992 l of R buffer to 8 l of 0.5M TCEP (use new tube). |
| - After MaxPar incubation, add 200 l of L buffer to each MaxPar tube and transfer to a 3 kDa Amicon Ultra filter unit. - Spin MaxPar-containing filter units at 12,000 x g for 25 minutes. Discard flow-through. | - After MaxPar-containing filter units start spinning, add 100 l of diluted TCEP to each antibody filter unit. Vortex briefly to mix. Incubate at 37 degrees C for exactly 30 minutes. - Start time: - End time: |
| - Add 400 l of C buffer to each MaxPar-containing filter unit. - Spin MaxPar-containing filter units at 12,000 x g for 25 minutes. Discard flow-through. | - After antibody incubation, add 300 l of C buffer. - Spin at 12,000 x g for 8 minutes. Discard flow through. - Add 400 l of C buffer to the antibody filter unit and spin at 12,000 x g for 8 minutes. |
| - Resuspend metal-loaded polymer in 200 l of C buffer. |  |
| - Transfer resuspended metal-loaded polymer to corresponding antibody tube. Mix briefly by vortexing. - Incubate at 37 degrees C for approximately 1.5 hours. - Start time: - End time: - Add 200 l of W buffer. Centrifuge at 12,000 x g for 8 minutes. Discard flow-through. - Add 400 l of W buffer. Centrifuge at 12,000 x g for 8 minutes. Discard flow-through. - Add 400 l of W buffer. Centrifuge at 12,000 x g for 8 minutes. Discard flow-through. - Add 400 l of W buffer. Centrifuge at 12,000 x g for 8 minutes. Discard flow-through. - Add 400 l of W buffer. Centrifuge at 12,000 x g for 8 minutes. Discard flow-through. - Add 400 l of W buffer. Centrifuge at 12,000 x g for 8 minutes. - Add 200 l of W buffer, pipette to mix and rinse the walls of the filter. Invert the column into a new collection tube. - Spin at 1000 x g for two minutes. - Add another 200 l of W buffer, pipette to mix and rinse the walls of the filter. Invert the column into the same collection tube. - Spin at 1000 x g for two minutes. - Use the Nanodrop to measure A280 (set to IgG mode) and record in antibodies table. - Dilute antibody to the highest round value (e.g. 0.5 mg/ml or 0.4 mg/ml) in W buffer with sodium azide or Candor Antibody Stabilization Solution. - Label and store at 4 degrees C. |  |

Comments